Revision for your Resit Test

Your third test is based on lectures 14 to 21. The actual test consists of 30 multiple choice questions only.

- A situation where several processes access and manipulate the same data concurrently and the outcome of the execution depends on the particular order in which access takes place is called ____________

- Data consistency

- Race condition

- Aging

- Starvation

- The segment of code in which the process may change common variables, update tables, write into files is known as ____________

- Program

- Critical section

- Non – critical section

- Synchronizing

- Which of the following conditions must be satisfied to solve the critical section problem?

- Mutual Exclusion

- Progress

- Bounded Waiting

- All of the above

- Mutual exclusion implies that ____________

- If a process is executing in its critical section, then no other process must be executing in their critical sections

- If a process is executing in its critical section, then other processes must be executing in their critical sections

- If a process is executing in its critical section, then all the resources of the system must be blocked until it finishes execution

- None of the above

- Bounded waiting implies that there exists a bound on the number of times a process is allowed to enter its critical section ____________

- After a process has made a request to enter its critical section and before the request is granted

- When another process is in its critical section

- Before a process has made a request to enter its critical section

- None of the above

- Semaphore is a/an  _______ to solve the critical section problem.

- Hardware for a system

- Special program for a system

- Integer variable

- None of the above

- What are the two kinds of semaphores?

- Mutex & counting

- Binary & counting

- Counting & decimal

- Decimal & binary

- The dining – philosophers problem will occur in case of ____________

- 5 philosophers and 5 forks

- 4 philosophers and 5 forks

- 3 philosophers and 5 forks

- 6 philosophers and 5 forks

- A monitor is a type of ____________

- Semaphore

- Low level synchronization construct

- High level synchronization construct

- None of the above

- The monitor construct ensures that ____________

- Only one process can be active at a time within the monitor

- n number of processes can be active at a time within the monitor (n being greater than 1)

- The queue has only one process in it at a time

- All of the above

- What are the operations that can be invoked on a condition variable?

- hold & wait

- Signal & hold

- Continue & signal

- Wait & signal

- Which of the following condition is required for a deadlock to be possible?

- Mutual exclusion

- A process may hold allocated resources while awaiting assignment of other resources

- no resource can be forcibly removed from a process holding it

- All of the above

- A system is in the safe state if ____________

- The system can allocate resources to each process in some order and still avoid a deadlock

- There exist a safe sequence

- All of the above

- None of the above

- The circular wait condition can be prevented by ____________

- Using thread

- Defining a linear ordering of resource types

- Using pipes

- All of the above

- A problem encountered in multitasking when a process is perpetually denied necessary resources is called ____________

- Deadlock

- Starvation

- Inversion

- Aging

- What is Address Binding?

- Going to an address in memory

- Locating an address with the help of another address

- Binding two addresses together to form a new address in a different memory space

- A mapping from one address space to another

- Binding of instructions and data to memory addresses can be done at ____________

- Compile time

- Load time

- Execution time

- All of the above

- If the process can be moved during its execution from one memory segment to another, then binding must be ____________

- Delayed until run time

- Preponed to compile time

- Preponed to load time

- None of the above

- What is Dynamic loading?

- Loading multiple routines dynamically

- Loading a routine only when it is called

- Loading multiple routines randomly

- None of the above

- What is the advantage of dynamic loading?

- A used routine is used multiple times

- An unused routine is never loaded

- CPU utilization increases

- All of the above

- If a higher priority process arrives and wants service, the memory manager can swap out the lower priority process to execute the higher priority process. When the higher priority process finishes, the lower priority process is swapped back in and continues execution. This variant of swapping is sometimes called?

- Priority swapping

- Pull out, push in

- Roll out, roll in

- None of the above

- The address generated by the CPU is referred to as ____________

- Physical address

- Logical address

- Neither physical nor logical

- None of the above

- Which of the following are forms of malicious attack?

- Theft of information

- Modification of data

- Wiping of information

- All of the above

- The address loaded into the memory address register of the memory is referred to as ____________

- Physical address

- Logical address

- Neither physical nor logical

- None of the mentioned

- The run time mapping from virtual to physical addresses is done by a hardware device called the ____________

- Virtual to physical mapper

- Memory management unit

- Memory mapping unit

- None of the above

- In contiguous memory allocation ____________

- Each process is contained in a single contiguous section of memory

- All processes are contained in a single contiguous section of memory

- The memory space is contiguous

- None of the above

- The relocation register helps in ____________

- Providing more address space to processes

- A different address space to processes

- To protect the address spaces of processes

- None of the above

- Physical memory is broken into fixed-sized blocks called ________

- Frames

- Pages

- Backing store

- None of the above

- Logical memory is broken into blocks of the same size called ________

- Frames

- Pages

- Backing store

- None of the above

- The size of a page is typically ____________

- Varied

- Power of 2

- Power of 4

- None of the above

- When the valid – invalid bit is set to valid, it means that the associated page ____________

- Is in the TLB

- Has data in it

- Is in the process’s logical address space

- Is the system’s physical address space

- The CPU hardware has a wire called __________ that the CPU senses after executing every instruction.

- Interrupt request line

- Interrupt bus

- Interrupt receive line

- Interrupt sense line

- The _________ determines the cause of the interrupt, performs the necessary processing and executes a return from the interrupt instruction to return the CPU to the execution state prior to the interrupt.

- Interrupt request line

- Device driver

- Interrupt handler

- All of the above

- In general the two interrupt request lines are ____________

- Maskable & non maskable interrupts

- Blocked & non maskable interrupts

- Maskable & blocked interrupts

- None of the above

- For large data transfers, _________ is used.

- DMA

- Programmed I/O

- Controller register

- None of the above

- What is the breach of integrity?

- This type of violation involves unauthorized reading of data

- This violation involves unauthorized modification of data

- This violation involves unauthorized destruction of data

- This violation involves unauthorized use of resources

- What is breach of confidentiality?

- This type of violation involves unauthorized reading of data

- This violation involves unauthorized modification of data

- This violation involves unauthorized destruction of data

- This violation involves unauthorized use of resources

- What is breach of availability?

- This type of violation involves unauthorized reading of data

- This violation involves unauthorized modification of data

- This violation involves unauthorized destruction of data

- This violation involves unauthorized use of resources

- What is known as a DOS attack?

- It is attacked to block traffic of network

- It is attacked to harm contents stored in HDD by worm spawn processes

- It is an attempt to make a machine or network resource unavailable

- None of the above

- What is Trojan horse?

- It is a useful way to encrypt password

- It is a user which steals valuable information

- It is a rogue program which tricks users

- It’s a brute force attack algorithm

- What is trap door?

- It is trap door in WarGames

- It is a hole in software left by designer

- It is a Trojan horse

- It is a virus which traps and locks user terminal

- Which mechanism is used by worm process?

- Trap door

- Fake process

- Spawn Process

- VAX process

- What is known as masquerading?

- When one participant in communication pretends to be someone else

- When attacker modifies data in communication

- When attack is of fraudulent repeat of a valid data

- When attack gains access to remote systems

- What is used to protect network from outside internet access?

- A trusted antivirus

- 24 hours scanning for virus

- Firewall to separate trusted and untrusted network

- Deny users access to websites which can potentially cause security leak

- Which direction access cannot happen using DMZ zone by default?

- Company computer to DMZ

- Internet to DMZ

- Internet to company computer

- Company computer to internet

- What are characteristic of Network Operating Systems?

- Users are aware of multiplicity of machines

- They are transparent

- They are simple to use

- All of the above

- What are the characteristics of Distributed Operating system?

- Users are aware of multiplicity of machines

- Access is done like local resources

- They have multiple zones to access files

- None of the above

- What are the characteristics of data migration?

- Transfer data by entire file or immediate portion required

- Transfer the computation rather than the data

- Execute an entire process or parts of it at different sites

- None of the above

- What are the characteristics of process migration?

A.Transfer data by entire file or immediate portion required
B. Transfer the computation rather than the data
C. Execute an entire process or parts of it at different sites
D. None of the above

- How many layers does the Internet model ISO consist of?

A.Three
B. Five
C. Seven
D. Eight